THE

SP-404A

FIELD MANUAL

Techniques, Cheats & Bank Architecture

Including the Tiger Dust Default Set Layout

Compiled for Jason Gronvold

Jambox Project  /  April 2026

NOT FOR RESALE — PERSONAL REFERENCE COPY

# Chapter 1: The Machine

The Roland SP-404A Linear Wave Sampler was released in 2017 as the successor to the SP-404SX. It carries forward the same 12-pad, bank-based architecture, 16-bit linear sampling, and 29 onboard DSP effects that defined the SP-404 sound. Just pads, knobs, and the effects engine that shaped an entire genre of music.

## 1.1 Hardware Overview

The 404A is built around a 3x4 pad grid (12 pads) with 10 banks (A through J), giving you 120 pad slots per SD card. Each pad holds one sample. Pads are not velocity-sensitive — every hit triggers at full velocity unless you adjust the pad’s volume setting.

Front Panel Quick Reference

| Control | Location | Function |
| --- | --- | --- |
| PAD GRID (1-12) | Center | Trigger samples. 3 rows of 4 pads. |
| SUB PAD | Below pad 12 | 13th pad — triggers an additional sample per bank |
| BANK A-J | Left of pads | Switch between 10 banks (120 pads + 10 sub pads) |
| CTRL 1-3 knobs | Top left | Real-time effect parameter control |
| VOLUME knob | Top right | Master output volume |
| REMAIN indicator | Top panel | LED shows remaining memory on current media |
| REC button | Right of pads | Arm sampling / resampling |
| PLAY / STOP | Right of pads | Transport controls |
| DEL button | Right of pads | Delete samples or cancel operations |
| MARK button | Right of pads | Set start/end points during sample editing |
| PATTERN SELECT | Below pads | Enter pattern record/playback mode |
| SD/SDHC card slot | Left side | Holds banks C-J. Supports up to 32GB SDHC. |
| MIC (built-in) | Top panel | Internal mic for quick sampling anywhere |
| INPUT L/R (RCA) | Rear | External audio input for sampling |
| MIC INPUT (1/4") | Rear | External microphone input |
| OUTPUT L/R (RCA) | Rear | Main audio output (RCA) |
| PHONES (1/4") | Front | Headphone output |
| MIDI IN | Rear | Receive MIDI from external gear |
| USB | Rear | File transfer to/from computer (Mini-USB) |
| POWER | Rear | 9V DC or 6x AA batteries |

NOTE:  The 404A uses RCA outputs. Keep RCA-to-1/4" adapters in your gig bag for connecting to mixers and audio interfaces.

## 1.2 Core Concepts

Sample Memory: Internal vs SD Card

Banks A and B live in the 404A’s internal memory. Banks C through J live on the SD card. If you swap SD cards, Banks A and B stay the same, but C-J change completely. Use A and B for your go-to utility sounds that you always want available regardless of which card is inserted.

Banks (A-J)

Each bank holds 12 pads plus 1 sub pad (13 samples per bank, 130 total across 10 banks). The working convention is: Pads 1-4 are drums (kick, snare, hat, percussion), Pads 5-8 are melodic elements (bass, chords, leads, pads), and Pads 9-12 are textures, loops, and effects. The sub pad is a wildcard — use it for a full beat loop, a transition effect, or whatever you reach for most often.

Polyphony: 12 Voices

The 404A can play a maximum of 12 samples simultaneously. If you trigger a 13th voice, the oldest playing sample gets cut. Long ambient pads and reverb tails eat into your polyphony budget — if your beat suddenly starts dropping sounds, you’ve hit the ceiling.

CHEAT:  Bake reverb into samples via resampling instead of using live reverb. A baked reverb tail stops when the sample ends. A live reverb effect holds the voice open until the tail decays.

Patterns

Patterns are sequenced arrangements of pad triggers recorded in real-time. The 404A quantizes to 16th notes by default. Patterns can be chained for longer arrangements.

Sample Format

16-bit WAV or AIFF at 44.1kHz. Maximum single sample length is approximately 180 minutes in stereo (limited by 2GB file size cap). The included Loopmasters SD card has over 100 factory samples.

HOW A SAMPLER BUILT A GENRE

In 1995, Roland released the MS-1 — a cheap, portable sampler that nobody at the company expected to change music. Three years later, the SP-202 arrived and hip-hop producers noticed. But it was the original SP-404, released in 2005, that detonated.

The timing was everything. In 2004, two albums had cracked open the underground: Madlib and MF DOOM’s Madvillainy and J Dilla’s Donuts. Both were sample-heavy, lo-fi, and defiantly uncommercial. A generation of bedroom producers heard those records and thought: I want to make that. The SP-404 was exactly what they needed — affordable, portable, and loaded with an effects engine that made everything sound like it had been dug out of a crate.

The lo-fi hip-hop community was born from the SP-404.

The effect that launched a thousand careers was Vinyl Sim. One button turned clean digital audio into something that sounded like it had lived on a dusty record shelf for thirty years. Pair it with the Lo-Fi effect — which crushed bit depth and sample rate into beautiful degradation — and suddenly anyone could make music that felt vintage, warm, and human.

By 2006, the Los Angeles beat scene was coalescing. Daddy Kev, DJ Nobody, and D-Styles launched Low End Theory, a weekly night at The Airliner in Lincoln Heights. The venue was small, the bass was enormous, and the SP-404 was the weapon of choice. Dibiase, Ras G, Flying Lotus, Samiyam, Tokimonsta — they all came up through that room, performing live sets on their 404s and 303s.

Flying Lotus took the SP-404’s effects processing into the studio and built Brainfeeder Records on the aesthetic. Ras G turned his apartment into Spacebase — a shrine of records, samplers, and Afrofuturist art where he made astral collages of laser-like blips fused with the spacey echo of Jamaican dub. When Ras G passed away in 2019, his family honored his love for the 404 by declaring April 4th (4/04) as Ras G Day. Roland eventually adopted the date as the annual "404 Day" celebration.

Meanwhile, in Tokyo, the same thing was happening independently. Hubs like Solfa Nakameguro hosted nights like 90 BPM Takeover, transplanting the Low End Theory spirit to Japan. Producers like Bugseed, Submerse, and Fumitake Tamura built a parallel scene. And in Shibuya, the late Nujabes — who owned two record shops and had founded Hydeout Productions in 1998 — was quietly creating the jazz-inflected, sample-heavy sound that would become the blueprint for lo-fi hip-hop on YouTube a decade later.

The SP-404 didn’t just play a role in these scenes. It defined their sonic identity. The crackle, the warmth, the slight pitch wobble, the layered degradation of chain-resampled audio — that IS the sound. No plugin has ever replicated it convincingly, because it’s not just the algorithm. It’s the converters, the gain staging, the specific way the hardware clips when you push it. It’s an instrument, not a tool.

— Compiled from Reverb, Sampleface, NPR, Roland Articles

# Chapter 2: Sampling Fundamentals

Getting audio into the SP-404A. Every method, every trick.

## 2.1 Recording from External Sources

Connect a turntable, synth, phone, or any audio source to the rear RCA inputs. Press REC, select a destination bank and pad, monitor the input level on the LED meter, and press the destination pad to start recording. Press it again to stop.

Input Level: Watch the LED meter. You want consistent activity without the peak LED lighting constantly. Aim for occasional peaks — the 16-bit format gives limited headroom, so do not clip.

Mono Sampling: Connect to the L/MONO input only for mono recording. This halves the file size.

Built-in Mic: The 404A has a small built-in mic on the top panel. Quality is limited, but it works for field recordings, room ambience, voice memos, and lo-fi vocal textures. Especially useful when running on batteries in the field.

## 2.2 Loading Samples via SD Card / USB

The 404A appears as a removable drive when connected via USB (Mini-USB cable). The SD card uses a specific folder structure — samples go in numbered folders corresponding to bank and pad positions. The Wave Converter software (free from Roland) lets you drag WAV files to specific pads.

Folder structure: On the SD card, samples are stored in the ROLAND/SP-404A/SMPL folder. Each sample is named by bank and pad position (e.g., A0000001.WAV for Bank A Pad 1).

Supported formats: 16-bit WAV and AIFF at 44.1kHz. Convert everything to this format before loading.

CHEAT:  Export Jambox samples as 16-bit 44.1kHz mono WAV when possible. Mono uses half the space. Reserve stereo for pads, ambient textures, and full beat loops where width matters.

## 2.3 Resampling (The Core Technique)

Resampling records the 404A’s own output — including the active effect — back onto a new pad. This is the defining creative technique. Each resample pass prints the effect and adds subtle hardware character.

The Resample Loop

Step 1: Load source material onto pads (vinyl rip, field recording, downloaded sample).

Step 2: Play a pad with an effect active (Vinyl Sim, Lo-Fi, Filter+Drive).

Step 3: Press REC and select an empty destination pad. The 404A records your output.

Step 4: Stop recording. Your performance + effect is now baked into the new pad.

Step 5: Trim the new sample’s start and end points.

Step 6: Repeat: play the resampled pad with a different effect, resample again.

Each pass adds compression artifacts, effect character, and subtle noise. After 3-4 rounds, you have that unmistakable layered, degraded SP-404 sound.

ADVANCED:  If you can get an SP-303, chain-resample between the two units. The 303’s vinyl compression has a grittier character. Recording from 404A to 303 and back adds a different flavor each pass.

## 2.4 Sample Editing

Without a waveform display, editing on the 404A is done entirely by ear. This forces you to develop precise listening skills.

Setting Start Point: Play the sample, press MARK when you hear where you want it to start. Audition and adjust with CTRL knobs.

Setting End Point: Same process for the end point. Listen for clicks at the loop point and nudge until clean.

BPM Sync: For loops, set the BPM so the sample syncs to the pattern tempo. Critical for keeping chopped breaks in time.

CHEAT:  Do precision editing in a DAW (Ableton, Audacity). Trim to exact bar lengths, normalize, export as 16-bit 44.1kHz WAV, load the pre-trimmed files onto the SD card. Use the 404A for performance and effects, not surgical editing.

DIBIASE AND THE ART OF CHAIN RESAMPLING

In the early 2000s, a producer named Dibiase was working out of Los Angeles with an Akai MPC2000XL and an SP-303. He developed a workflow that would become the textbook for an entire generation of beatmakers.

The technique was simple in concept, radical in execution: make a beat on the MPC, dump it into the SP-303, resample it through Vinyl Sim, then dump it back into the 404. Then do it again. And again. Each pass through the hardware added a different layer of degradation — the 303’s vinyl compression was grittier, the 404’s was warmer. The converters colored the sound differently each time. Three or four round trips and the beat sounded like it had been excavated from a time capsule.

A lot of producers make their beats on the 404 and then double compress it by dumping it into the 303 and back. A lot of bouncing back and forth.

Dibiase didn’t invent resampling. But he turned it into a philosophy. Where other producers treated the SP-404 as a playback device — load samples, trigger them, record the output — Dibiase treated the machine itself as an instrument with its own voice. The degradation wasn’t a side effect. It was the whole point.

When Roland began developing the MK2, Dibiase was one of the artists they consulted. They wanted to understand how producers actually used the machine versus how Roland had designed it to be used. The answer was: nothing like they expected. The resample workflow, the effect stacking through successive generations, the deliberate abuse of the hardware’s limitations — none of that was in the manual. It was folk knowledge, passed from producer to producer through beat battles, forum posts, and late-night sessions.

On the 404A, you have the same core ingredients. One effect at a time. Resample to bake it. Stack effects through generations. The constraints are identical, which means the technique works exactly the same way. The only difference is that you’re doing it on a machine with 12 pads instead of 16 — which just means you fill up banks faster and swap more often. Some producers consider that a feature, not a limitation.

— Based on Micro-Chop interviews, SP-Forums, Gearspace

# Chapter 3: The 29 Effects

The SP-404A’s effects engine is its soul. These are colored, aggressive, character-defining tools. One effect active at a time, controlled by three knobs. That constraint forces you to commit.

## 3.1 Complete Effects Reference

| # | Effect | CTRL 1 | CTRL 2 | CTRL 3 | Notes |
| --- | --- | --- | --- | --- | --- |
| 1 | Filter+Drive | Cutoff | Resonance | Drive | Low-pass filter with saturation. The workhorse. |
| 2 | Pitch | Pitch | Fine | Feedback | Real-time pitch shifting. |
| 3 | Delay | Time | Feedback | Level | Tempo delay. Automate time for dub throws. |
| 4 | Isolator | Low | Mid | High | 3-band kill EQ. Cut bands to zero for drops. |
| 5 | Vinyl Sim | Noise | Wow | Tone | THE SP-404 effect. Crackle, wobble, warmth. |
| 6 | Reverb | Time | Tone | Level | Room/hall. Heavy on snares, light on kicks. |
| 7 | Tape Echo | Time | Feedback | Level | Warm analog-style delay with tape character. |
| 8 | Chorus | Rate | Depth | Level | Subtle width and detuning. |
| 9 | Flanger | Rate | Depth | Feedback | Jet sweep. Metallic on drums. |
| 10 | Phaser | Rate | Depth | Feedback | Phase shifting. Psychedelic on pads. |
| 11 | Tremolo/Pan | Rate | Depth | Wave | Volume modulation or auto-pan. |
| 12 | Distortion | Drive | Tone | Level | Hard clipping. Riot Mode territory. |
| 13 | Overdrive | Drive | Tone | Level | Softer saturation than Distortion. |
| 14 | Fuzz | Sustain | Tone | Level | Thick, buzzy, guitar-pedal fuzz. |
| 15 | Wah | Freq | Depth | Rate | Auto-wah or manual with CTRL 1. |
| 16 | Octave | Octave | Direct | Effect | Pitch down one octave. Instant sub bass. |
| 17 | Compressor | Thresh | Ratio | Level | Dynamics control. Glue your drums. |
| 18 | Equalizer | Low | Mid | High | 3-band EQ for tone shaping. |
| 19 | Lo-Fi | Bit | Rate | Filter | Bit crush + sample rate reduction. |
| 20 | Noise Gen | Color | Level | Filter | Generate noise textures. Layer over beats. |
| 21 | Radio Tuning | Freq | Noise | Level | Simulates tuning between stations. |
| 22 | Slicer | Rate | Depth | Pattern | Rhythmic gating. Stutter patterns. |
| 23 | Ring Mod | Freq | Depth | Level | Metallic, atonal, experimental. |
| 24 | Chromatic PS | Pitch | Fine | Level | Chromatic pitch shift. Play melodies. |
| 25 | Voice Trans | Pitch | Formant | Level | Vocal transformation. Robot to chipmunk. |
| 26 | C.Canceler | Freq | Width | Level | Center cancellation. Remove vocals. |
| 27 | Subsonic | Freq | Level | Decay | Add sub bass frequencies underneath. |
| 28 | BPM Looper | Length | Gate | Level | Tempo-synced loop capture and repeat. |
| 29 | DJFX Looper | Speed | Gate | Level | Real-time stutter/scratch. |

## 3.2 The Essential Five

You could make entire albums with just these five. Master them first.

Vinyl Sim (#5)

CTRL 1 adds vinyl surface noise (crackle). CTRL 2 adds wow and flutter (pitch wobble). CTRL 3 adjusts tone (bright vs warm). At low settings: subtle warmth. At high settings: destroyed record. Most lo-fi producers keep Noise at 30-40%, Wow at 15-25%, Tone at noon.

RECIPE:  Vinyl Sim at Noise 35%, Wow 20%, Tone noon. Resample through this once and everything gets that dusty character.

Lo-Fi (#19)

CTRL 1 crushes bit depth. CTRL 2 reduces sample rate. CTRL 3 filters the harshness. Subtle on drums = instant vintage. Heavy = industrial noise.

Filter+Drive (#1)

CTRL 1 sweeps the cutoff — your most expressive real-time control. Close to darken, open to brighten. CTRL 3 adds saturation. This effect alone covers half of what you need for live performance.

DJFX Looper (#29)

Captures a tiny slice of audio and loops it at the speed set by CTRL 1. Tap the effect on a drum break for stutter fills. Slow for tape-stop effects. Fast for glitch.

Isolator (#4)

Three-band kill EQ. Turn a knob fully left to kill that band entirely. Cut the lows before a breakdown, then slam them back in. Kill the mids for a telephone effect.

## 3.3 Stacking Effects via Resampling

The 404A only allows one effect at a time. The workaround is resampling: apply Effect A, resample to a new pad. Play the new pad with Effect B, resample again. Each pass stacks effects through successive generations.

CHEAT:  Want Vinyl Sim AND Lo-Fi? Play with Vinyl Sim, resample. Play the result with Lo-Fi, resample again. Two effects baked into one pad.

## 3.4 Recommended Effect Chains per Jambox Bank

Primary effect applied first, secondary stacked via resample:

| Bank | Primary Effect | Secondary (resample) | Character |
| --- | --- | --- | --- |
| Tiger Dust | Vinyl Sim | Lo-Fi (light) | Classic dusty aesthetic |
| Riot Mode | Distortion / Fuzz | Compressor (smash) | Aggressive punk energy |
| Minneapolis Machine | Lo-Fi | Tape Echo | Dusty boom-bap, experimental |
| Brat Mode | Lo-Fi (heavy) | Slicer / Ring Mod | Glitchy, chaotic, processed |
| Big Beat | Compressor | DJFX Looper | Fat breaks, rave energy |
| Free Essentials | Vinyl Sim (light) | Reverb | Clean utility |

SPACEBASE WAS THE PLACE

Gregory Shorter Jr. — Ras G — lived and worked in a small Los Angeles apartment he called Spacebase. The walls were covered with Afrofuturist art. Shelves overflowed with vinyl. Electronic samplers occupied every flat surface. An SP-404 sat at the center of it all.

Ras G made astral collages: laser-like blips and squeals fused with decades of bass-heavy beat music, the spacey echo of Jamaican dub, and the cosmic jazz of Sun Ra. His tools were simple — records, a turntable, and the 404. He’d sample a Sun Ra record, run it through Vinyl Sim and Reverb via resample stacking, pitch it down with the Octave effect, and layer it over a drum break until it sounded like transmissions from another galaxy.

His apartment was his brain. He called it Spacebase.

At Low End Theory, Ras G’s live sets were legendary. He’d stand behind the 404, triggering pads and riding effects in real-time, building beats that felt like they were being channeled rather than performed. The bass would shake the walls of The Airliner. The crowd would lose their minds.

Ras G passed away on July 29, 2019, at the age of 40. His family honored his love for the 404 by declaring April 4th as Ras G Day and releasing his posthumous albums. Roland eventually adopted the date as the annual 404 Day celebration — a global event honoring beat culture and the machine that enabled it.

The lesson from Ras G is the lesson of the 404 itself: constraints breed creativity. He didn’t need a studio full of gear. He needed records, pads, knobs, and the willingness to treat the machine as a collaborator rather than a tool. Everything that came out of Spacebase was proof that limitation is a feature.

— Based on NPR, Reverb, Sampleface

# Chapter 4: Patterns & Performance

## 4.1 Real-Time Pattern Recording

The 404A records patterns in real-time. Press PATTERN SELECT, choose a pattern slot, press REC, and play your pads. The 404A captures your timing. Default quantize is 16th notes.

Quantize settings: Change quantize resolution or turn it off entirely for loose, human-feel patterns. Quantize off is how you get that "drunk" SP-404 groove.

Overdub: Layer additional parts by recording over an existing pattern. Each pass adds new triggers without erasing old ones. Build up: drums first, then bass, then melodic, then textures.

CHEAT:  Record kick and snare with quantize ON (tight grid). Overdub hi-hats and percussion with quantize OFF (loose feel). This mix of tight and loose is the secret sauce.

## 4.2 Pattern Chains

Chain multiple patterns into longer arrangements. Pattern 1 (intro) flows into Pattern 2 (verse) into Pattern 3 (chorus). The 404A plays them in sequence.

## 4.3 Live Performance Techniques

Effect Switching

The 404A switches effects smoothly — no gap or click. Ride effects in real-time: start on Vinyl Sim, switch to Filter+Drive for a sweep, jump to DJFX Looper for a stutter, back to Vinyl Sim. Practice the physical motion.

Muting and Unmuting

During pattern playback, mute individual pads by holding DEL and pressing the pad. Unmute by pressing again. This gives live arrangement control — drop drums for a breakdown, bring in bass, slam everything back.

Bank Switching

Switch banks mid-performance to access different sound sets. With 12 pads per bank and 10 banks, you have 120+ sounds available without stopping.

Resampling as a Live Looper

Use resampling as a live looper: start recording to an empty pad, play your performance, stop recording. The resampled pad now contains your loop. Trigger it and layer more on top.

PRO TIP:  Keep Banks I and J empty during performances. Use them as scratch pads for live resampling. When they fill up, you have a recording of your best moments.

## 4.4 Playing with a Band / DJ Set

The 404A’s RCA outputs connect directly to a DJ mixer. Route it into a channel alongside turntables or CDJs. Use the 404A for sample drops, stutter effects, and live beat layering.

Sync: No automatic tempo sync. You sync by ear or by tapping BPM. Practice matching your pattern tempo to the music alongside.

MIDI: The 404A receives MIDI IN. You can trigger pads from an external MIDI controller or sequencer.

FROM SHIBUYA TO THE WORLD

The SP-404 was born in Japan, but its first spiritual home outside Roland’s engineering labs was a record store in Shibuya.

Jun Seba — Nujabes — owned two shops in the neighborhood: T Records and Guinness Records. In 1998 he founded Hydeout Productions and began producing the atmospheric, jazz-inflected instrumental hip-hop that would later be recognized as the blueprint for an entire YouTube subculture. His sound was warm, melodic, and deeply rooted in jazz harmony — Coltrane samples layered over booming breakbeats, Rhodes piano floating above dusty drum loops. The SP’s effects engine was central to that warmth.

Nujabes passed away in a car accident in 2010. But his influence only grew. When lo-fi hip-hop study streams exploded on YouTube in the mid-2010s, the template was unmistakably his: jazz samples, gentle beats, vinyl texture. The SP-404 was the tool that made that texture accessible to thousands of bedroom producers worldwide.

Beatmakers around the world use hardware birthed in Japan to create these beats. And Japan’s own community was growing right alongside them.

In Tokyo, venues like Solfa Nakameguro hosted nights like 90 BPM Takeover, transplanting the Low End Theory spirit to Japan. A new generation of Japanese beatmakers — Bugseed, Submerse, Fumitake Tamura — strengthened the global reputation of their scene. The SP-404 was the common thread connecting producers in LA, Tokyo, London, and São Paulo, all making music that sounded like it came from the same dusty room even though they were separated by oceans.

That’s the thing about the SP-404: it doesn’t just process audio. It imposes a sonic fingerprint. Two producers on opposite sides of the world, sampling completely different source material, running it through Vinyl Sim and Lo-Fi — their beats will share a family resemblance. The machine is the genre.

— Based on Roland Articles, Yokogao Magazine, Sabukaru

# Chapter 5: Tiger Dust Default Set

Tiger Dust is the flagship Jambox bank — curated from Jason’s personal listening library. The YouTube playlist mining extracted 123 tracks revealing 9 genre clusters: synth-pop, post-punk, big beat, experimental hip-hop, electro-clash, riot grrrl, industrial, ambient, and noise. Purity Ring and P.O.S tied at 7 appearances each as the most-represented artists.

This set uses 8 of 10 banks, leaving 2 open for live resampling. Banks A-B are internal memory (always available regardless of SD card). Banks C-H are on the SD card.

## 5.1 Bank Map Overview

| Bank | Memory | Role |
| --- | --- | --- |
| A | Internal | Drums & Hits — your go-to kit (survives card swaps) |
| B | Internal | Transitions & Utility — always-on tools (survives card swaps) |
| C | SD Card | Bass & Low End |
| D | SD Card | Synths & Melodic |
| E | SD Card | Vocals & Chops |
| F | SD Card | Textures & Ambient |
| G | SD Card | Full Loops & Beats |
| H | SD Card | Tiger Dust Wildcards |
| I | SD Card | (Empty — Live Resample Scratch) |
| J | SD Card | (Empty — Live Resample Scratch) |

STRATEGY:  Banks A and B are internal memory — they survive SD card swaps. Your most essential sounds live here.

## 5.2 Pad Layouts

Bank A: Drums & Hits (Internal)

Your primary drum kit. These 12 sounds cover 80% of beat-making needs.

| A1 Kick — punchy | A2 Kick — dusty/lo-fi | A3 Snare — crack | A4 Snare — lo-fi |
| --- | --- | --- | --- |
| A5 Clap — layered | A6 Rim shot | A7 Hi-hat — closed | A8 Hi-hat — open |
| A9 Crash cymbal | A10 Perc — shaker | A11 Perc — conga | A12 Break — choppable |

WORKFLOW:  Pads A1-A8 for finger drumming. Pad A12 is a full break — play it as a bed, layer live hits from A1-A8 on top via resampling.

Bank B: Transitions & Utility (Internal)

Sounds you reach for in every session regardless of genre.

| B1 Vinyl crackle bed | B2 Tape hiss bed | B3 Impact — boom | B4 Reverse cymbal |
| --- | --- | --- | --- |
| B5 Riser — sweep up | B6 Downer — sweep down | B7 Record scratch | B8 DJ horn/siren |
| B9 Silence (mute pad) | B10 Count-in (4 clicks) | B11 Room tone | B12 Feedback swell |

CHEAT:  Pad B9 is intentional silence. Trigger it on a pattern step to create a mute/gap. Old-school trick for rhythmic space.

Bank C: Bass & Low End

| C1 Sub bass — C root | C2 Sub bass — E root | C3 Sub bass — G root | C4 808 bass — long decay |
| --- | --- | --- | --- |
| C5 808 bass — short | C6 Synth bass — dirty | C7 Synth bass — round | C8 Picked bass sample |
| C9 Upright bass sample | C10 Bass loop — 100bpm | C11 Bass loop — 120bpm | C12 Bass drone — dark |

CHEAT:  Pads C1-C3 are tuned to C, E, G — a minor triad. Use Chromatic PS (Effect #24) to pitch them into any key.

Bank D: Synths & Melodic

| D1 Chord stab — minor | D2 Chord stab — major | D3 Chord — 7th | D4 Pad — warm wash |
| --- | --- | --- | --- |
| D5 Lead — saw | D6 Lead — square | D7 Arpeggio — 16th | D8 Electric piano hit |
| D9 Organ stab | D10 Strings — short | D11 Synth riser | D12 Synth fall/sweep |

Bank E: Vocals & Chops

| E1 Vocal chop 1 | E2 Vocal chop 2 | E3 Vocal chop 3 | E4 Vocal chop 4 |
| --- | --- | --- | --- |
| E5 Shout — "hey" | E6 Shout — "yeah" | E7 Breath texture | E8 Spoken word clip |
| E9 Vocal loop — airy | E10 Vocal loop — chopped | E11 Harmonized stack | E12 Vocal noise/glitch |

Bank F: Textures & Ambient

| F1 Rain / nature | F2 City ambience | F3 Room tone — large | F4 Machine hum |
| --- | --- | --- | --- |
| F5 Glitch texture | F6 Noise burst — short | F7 Static bed | F8 Radio tuning clip |
| F9 Drone — ominous | F10 Drone — warm | F11 Tape warble | F12 Found sound / foley |

Bank G: Full Loops & Beats

| G1 Full beat — 90bpm | G2 Full beat — 100bpm | G3 Full beat — 120bpm | G4 Full beat — 140bpm |
| --- | --- | --- | --- |
| G5 Drum break — raw | G6 Drum break — dusty | G7 Drum break — fast | G8 Melodic loop — keys |
| G9 Melodic loop — synth | G10 Vocal loop — hook | G11 Ambient loop | G12 Noise loop |

WORKFLOW:  Trigger a full beat from G1-G4, layer sounds from other banks on top. When it sounds right, resample to Bank I or J.

Bank H: Tiger Dust Wildcards

Genre-crossing sounds that don’t fit neatly into one category. The sounds that make Tiger Dust sound like Tiger Dust.

| H1 Purity Ring-style pad | H2 P.O.S-style vocal chop | H3 Post-punk guitar riff | H4 Electro-clash stab |
| --- | --- | --- | --- |
| H5 Big beat break chop | H6 Industrial metal hit | H7 Detuned synth wobble | H8 Reversed melody |
| H9 Bit-crushed chord | H10 Filtered vocal phrase | H11 Distorted piano | H12 Mystery sound |

PRO TIP:  Bank H is your secret weapon. Drop a Purity Ring pad over a punk break and you get something nobody else has.

Banks I-J: Empty (Scratch Pads)

Left intentionally empty for live resampling, session captures, and experiments. Before the next session, review what you captured and promote the best sounds into Banks A-H.

KNXWLEDGE AND THE INFINITY OF VINYL

Glen Boothe — Knxwledge — is one of the most prolific beatmakers alive. His Bandcamp page has hundreds of releases. His Instagram is a constant stream of short videos: a record spinning on a turntable, fingers on SP-404 pads, a new beat materializing in real-time. The whole process takes minutes.

His workflow is deceptively simple. He digs for vinyl — soul, funk, jazz, R&B, anything with a moment worth capturing. He samples a few seconds into the 404. He chops. He plays the chops with Vinyl Sim engaged. He resamples. The beat exists. On to the next one.

The volume of output is the point. Knxwledge treats beat-making like a practice, not a production. The way a jazz musician plays scales every morning, Knxwledge makes beats every day. Most of them go straight to Bandcamp for a dollar. Some of them end up on Anderson .Paak albums. The quality is consistent because the practice is constant.

I have this weird, wild appreciation for sampling.

His Meek Mill remix series became legendary: he’d take Meek’s most aggressive verses and lay them over gentle soul and funk samples running at half the original tempo. The contrast was startling and beautiful. The SP-404 was the glue — its effects engine smoothed the edges between the raw vocal energy and the soft instrumental bed.

The lesson from Knxwledge is about volume and trust. Trust the machine. Trust the process. Don’t overthink. Sample, chop, effect, resample, move on. The 404 rewards speed and instinct over deliberation. The best beats often come from the fastest sessions.

— Based on Bandcamp Daily, Sampleface, HYPEBEAST

# Chapter 6: Cheat Codes

Shortcuts, hidden moves, and time-savers.

## 6.1 Button Combinations

| Combo | What It Does |
| --- | --- |
| REC + PAD (no input source) | Resample internal output to that pad |
| DEL + PAD (during playback) | Mute/unmute that pad in the pattern |
| MARK during recording | Set a mark point for later editing |
| Hold CANCEL while selecting effect | Preview effect without committing |
| BANK + PAD simultaneously | Quick-jump to specific bank+pad combo |
| Hold PAD to preview | Audition a pad without full trigger |
| DEL + MARK on a sample | Delete the sample from that pad |
| PATTERN + number pads | Select pattern slots for chaining |

## 6.2 Sample Prep Rules

| Rule | Why |
| --- | --- |
| 16-bit 44.1kHz WAV (always) | Native format. No conversion needed. |
| Trim all silence before loading | Tight samples = instant response. No lag. |
| Normalize to -1dB | Consistent volume across all pads. |
| Name files: BPM_KEY_description.wav | Makes organizing on the SD card easier. |
| One-shots under 3 seconds | Quick trigger. Saves space. Stays in polyphony budget. |
| Loops at exact bar lengths | 2 bars, 4 bars, 8 bars. Locks to pattern grid. |
| Mono for one-shots, stereo for pads/loops | Halves file size on one-shots. |
| Keep SD card under 16GB of content | Larger cards work but navigation slows. |

## 6.3 Performance Cheats

| Technique | How |
| --- | --- |
| Fake sidechain pump | Resample bass pad through Compressor with kick playing. Compressor ducks on kicks. Bake it. |
| Instant lo-fi | Vinyl Sim: Noise 35%, Wow 20%, Tone noon. One resample pass. |
| Stack effects | Apply Effect A, resample. Apply Effect B to result, resample. Repeat. |
| Stutter fill | DJFX Looper on a drum break. Tap rhythm on CTRL knob. |
| Fake vinyl stop | Play a loop, switch to Pitch effect, slowly turn CTRL 1 down. |
| Mute gaps | Use silence pad (B9) on pattern steps for rhythmic space. |
| Layer thickness | Trigger kick + sub bass on same pattern step. |
| Live arrangement | Start all pads unmuted. DEL+PAD to strip layers. Build back up. |
| Quick vocal chop | Resample vocal with BPM Looper to capture slices. |

## 6.4 SD Card Management

Backup before every session: Copy the entire SD card to your computer. No undo for deleted samples.

Dedicated cards: Separate SD cards for different projects. Label them. Swap cards = swap worlds.

Factory card: Keep the Loopmasters card as-is. Never overwrite it.

Formatting: Format new SD cards IN the SP-404A to ensure correct folder structure.

CHEAT:  Keep one card as your "Tiger Dust Performance" card with the default set locked in. Keep another as your "Studio Experiments" card. Never mix them.

# Chapter 7: Jambox Pipeline

Quick reference for the automated sample management system feeding the SP-404A.

## 7.1 Ingest Pipeline

1. File Watcher: Detects new files in the ingest folder.

2. Format Normalize: Convert to 16-bit 44.1kHz WAV. Strip silence. Normalize to -1dB.

3. Essentia Analysis: Auto-extract BPM, key, energy, genre, mood, loudness.

4. CLAP Bank Scoring: Score sample against all bank descriptions via natural language similarity.

5. LLM Decision: Qwen3 8B receives features + scores + context. Outputs: bank, pad position, vibe tags, quality rating.

6. Metadata Write: Tags written to file. OneTagger cross-references MusicBrainz/Spotify.

7. Library Write: File moved to correct bank folder. _SOURCES.txt sidecar updated.

## 7.2 Tool Stack

| Tool | Role | Install |
| --- | --- | --- |
| Essentia | BPM, key, genre, mood analysis | pip install essentia |
| CLAP (LAION) | Zero-shot audio classification | pip install laion-clap |
| audio-separator | Stem splitting (Demucs, Mel-Roformer) | pip install audio-separator |
| OneTagger | Bulk file tagging | onetagger.github.io |
| madmom | Beat/downbeat detection | pip install madmom |
| librosa | Spectral analysis | pip install librosa |
| Qwen3 8B | LLM intelligence via Ollama | ollama pull qwen3:8b |
| ChromaDB | Vector store for CLAP embeddings | pip install chromadb |

## 7.3 404A-Specific Export

The pipeline’s final step converts organized samples into SP-404A-ready SD card format:

SD Card Structure: ROLAND/SP-404A/SMPL/ with files named by bank+pad position.

12 pads per bank: Pipeline respects the 12-pad limit. Overflow goes to next bank.

Internal vs SD: Banks A-B map to internal memory. Banks C-J to SD card.

Pad assignment: Pads 1-4 = drums/rhythm, 5-8 = melodic, 9-12 = texture/loops.

GOAL:  The pipeline should output a ready-to-copy SD card folder. Drop on card, insert in 404A, play.

# Chapter 8: Resources

## 8.1 Documentation

SP-404A Owner’s Manual: static.roland.com/assets/media/pdf/SP-404A_eng01_W.pdf

SP-404A Reference Manual: files.kraftmusic.com/media/ownersmanual/Roland_SP-404A_Reference_Manual.pdf

Roland Support: roland.com/global/support/by_product/sp-404a/owners_manuals

## 8.2 Communities

SP-Forums: sp-forums.com — The original SP-404 community. Tips, tricks, sample sharing.

Reddit r/SP404: reddit.com/r/SP404 — Active community, beat showcases, all models.

## 8.3 Sample Pack Sources

SPVIDZ: spvidz.com — Lo-fi drum packs processed through real SP-404 hardware.

LoFi Weekly: lofiweekly.com — Free weekly sample packs.

Electronisounds: electronisounds.com — One-shot focused collections.

NearTao: neartao.wordpress.com — Free session packs recorded on SP hardware.

## 8.4 Producers to Study

Dibiase: Chain-resampling pioneer. SP-303/404 dual workflow. The textbook.

Ras G: Afrofuturist lo-fi. Spacebase. The spiritual heart of the 404 community.

Flying Lotus: SP-404 effects processing in studio productions. Brainfeeder founder.

Knxwledge: Prolific vinyl sampler. Volume over perfection. Anderson .Paak collaborator.

Nujabes: Jazz-inflected instrumental hip-hop. The blueprint for lo-fi YouTube.

The Alchemist: Uses SP-404 as texture/effects tool alongside MPC.

STLNDRMS: Controllerise collective. MPC + SP-303 + SP-404 hybrid.